Emission standard of air pollutants for hazardous waste incineration

Abstract：

1. This document specifies the process performance requirements for pollution control, emission control requirements, pollutant monitoring requirements, compliance judgment, as well as implementation and supervision of air pollutants from existing hazardous waste incineration facilities (excluding medical waste and polychlorinated biphenyl waste).

2. The process performance indicators for hazardous waste incineration facilities are as follows：

| Indicator | Temperature in high-temperature zone of incinerator, ℃ | Flue gas residence time, s | Oxygen content in flue gas, % (stack sampling port) | Carbon monoxide concentration in flue gas, mg/m3 (stack sampling port) |  | Combustion efficiency, % | Destruction removal efficiency, % | Loss of ignition, % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Limit Value | ≥1100 | ≥2.0 | 6~15 | Hourly average value | Daily average value | ≥99.9 | ≥99.99 | ＜5 |
|  |  |  |  | ≤50 | ≤40 |  |  |  |

3. The emission limits for flue gas pollutants from hazardous waste incineration facilities are as follows:

Unit: mg/m3

| Serial Number | Pollutant Items | Limit Valuea | Sample Duration |
| --- | --- | --- | --- |
| 1 | Particulate matterb | 20 | Hourly average value |
|  |  | 15 | Daily average |
| 2 | Carbon monoxide (CO) | 50 | Hourly average value |
|  |  | 40 | Daily average |
| 3 | Nitrogen oxides (NOX) | 250 | Hourly average value |
|  |  | 200 | Daily average |
| 4 | Sulfur dioxide (SO2) | 100 | Hourly average value |
|  |  | 50 | Daily average |
| 5 | Hydrogen fluoride (HF) | 2.0 | Hourly average value |
|  |  | 1.0 | Daily average |
| 6 | Hydrogen chloride (HCl) | 50 | Hourly average value |
|  |  | 10 | Daily average |
| 7 | Ammoniac | 8 | Hourly average value |
| 8 | Mercury and its compounds (calculated as Hg) | 0.05 | Determination Average |
| 9 | Thallium and its compounds (calculated as Tl) | 0.02 | Determination Average |
| 10 | Cadmium and its compounds (calculated as Cd) | 0.03 | Determination Average |
| 11 | Lead and its compounds (calculated as Pb) | 0.5 | Determination Average |
| 12 | Arsenic and its compounds (calculated as As) | 0.5 | Determination Average |
| 13 | Chromium and its compounds (calculated as Cr) | 0.4 | Determination Average |
| 14 | Tin, antimony, copper, manganese, nickel, cobalt and their compounds (calculated as Sn+Sb+Cu+Mn+Ni+Co) | 0.6 | Determination Average |
| 15 | Dioxins（ng TEQ/Nm3）d | 0.1 | Determination Averaged |
| aThe pollutant limit value is the emission concentration at the baseline oxygen content. bDuring the start-up, shutdown, and shutdown periods caused by faults or accidents, the hourly average limit value of particulate matter is 150 mg/m3. cApplicable to hazardous waste incineration facilities that adopt SNCR and SCR to control flue gas nitrogen oxides. dThe arithmetic average of three consecutive determination values. |  |  |  |